THIN SECTIONS DESCRIPTION. WELL J-ROC 1

Amsden Formation (Core Plugs)

Energy & Environmental Research Center | 15 North 23rd Street, Stop 9018 | Grand Forks, ND 58202-9018

## PHOTOMICROGRAPHS

19 thin sections were described in Amsden Formation in the well J-ROC 1. The description was carried out based on the grain size, shape, cement/matrix type, mineral components, diagenesis, and depositional environment. An average of 30 photos were taken for each thin section where some of them were selected to illustrate the main mineral components, fractures, porosity, and type of contacts between grains in each sample.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130662

Well: J-ROC 1

Depth: 4978 ft

Grain size: 10 to 50 μm (quartz)

Grain shape: Subrounded to subangled

Sorting: NA

Matrix: Dolomite

Diagenesis: This sample is sandy-dolomite.  The sample is mainly composed by dolosparite and quartz. Feldspar, clay, and anhydrite are present. The presence of iron oxides is observed. The observed porosity is mainly due to the dissolution of quartz and felspar and averages 5%.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130663

Well: J-ROC 1

Depth: 4983 ft

Grain size: 10 to 150 μm

Grain shape: Subrounded to subangled

Sorting: Moderately sorted

Cement: Dolomite

Diagenesis: This sample is a sandstone. It is composed by quartz and feldspar. The presence of dolomite and anhydrite is noticeable as cements. The contact between grains is tangential and sometimes separated by the dolomitic and anhydritic cements. The observed porosity is intercrystalline and due also to the dissolution of feldspar and quartz and averages 8%.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130664

Well: J-ROC 1

Depth: 4991.5 ft

Grain size: 10 to 150 μm

Grain shape: Subrounded to subangled

Sorting: Moderately sorted

Cement: Dolomite and anhydrite

Diagenesis: This sample is a dolo-sandstone. It is composed by quartz, feldspar, and dolomite as cement. The contact between grains is tangential and separated by the dolomitic cements. The presence of clay and iron oxides is visible. The observed porosity is due to the dissolution of feldspar and quartz and averages 8%.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130665

Well: J-ROC 1

Depth: 4997 ft

Grain size: 10 to 50 μm (quartz)

Grain shape: Subrounded to subangled

Sorting: NA

Matrix: Dolomite

Diagenesis: This sample is dolomite.  The sample is mainly composed by dolosparite and quartz. Feldspar, and anhydrite are also present. The presence of iron oxides is detected. The observed porosity is mainly due to the dissolution of quartz and felspar and averages 2%.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130666

Well: J-ROC 1

Depth: 5002 ft

Grain size: 10 to 250 μm

Grain shape: Subrounded to subangled

Sorting: Moderately sorted

Cement: Dolomite

Diagenesis: This sample is a sandstone. It is composed by quartz and feldspar. The presence of dolomite is noticeable as cement. The contact between grains is tangential and separated also by the dolomitic cement. The observed porosity is intercrystalline and due also to the dissolution of feldspar and quartz and averages 30%.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130667

Well: J-ROC 1

Depth: 5006 ft

Grain size: 10 to 300 μm

Grain shape: Subrounded to subangled

Sorting: Moderately sorted

Cement: Dolomite

Diagenesis: This sample is a sandstone. It is composed by quartz and feldspar. The presence of dolomite is noticeable as cement. The presence of iron oxides is visible. The contact between grains is tangential and separated also by the dolomitic cement. The observed porosity is intercrystalline and due also to the dissolution of feldspar and quartz and averages 15%.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130668

Well: J-ROC 1

Depth: 5008 ft

Grain size: 10 to 200 μm

Grain shape: Subrounded to subangled

Sorting: Moderately sorted

Cement: Dolomite

Diagenesis: This sample is a dolo-sandstone. It is composed by quartz, feldspar, and dolomite. The presence of dolomite is noticeable as cement and as thin layers. The presence of iron oxides is visible. The quartz minerals show overgrowth. The contact between grains is tangential and separated also by the dolomitic cement. The observed porosity is intercrystalline and due also to the dissolution of feldspar and quartz and averages 12%.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130669

Well: J-ROC 1

Depth: 5012 ft

Grain size: NA

Grain shape: NA

Sorting: NA

Matrix: Anhydrite

Diagenesis: This sample is an anhydrite. It is composed mainly by anhydrite minerals. The presence of dolomite and iron oxides is observable and highlight the filled fractures. The observed porosity is almost null.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130670

Well: J-ROC 1

Depth: 5020 ft

Grain shape: Subrounded to subangled

Sorting: Moderately sorted

Cement: Anhydrite

Diagenesis: This sample is a sandstone. It is composed by quartz and feldspar with the presence of anhydrite as cement. The quartz minerals show overgrowth. The presence of iron oxides is observable. The contact between grains is tangential and mainly separated by the anhydritic cement. The observed porosity is null.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130671

Well: J-ROC 1

Depth: 5025 ft

Grain shape: Subrounded to subangled

Sorting: Moderately sorted

Cement: Anhydrite

Diagenesis: This sample is a sandstone. It is composed by quartz and feldspar with the presence of anhydrite and dolomite as cements. The presence of iron oxides is observable. The contact between grains is tangential and mainly separated by the anhydritic and dolomitic cements. The observed porosity is null.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130672

Well: J-ROC 1

Depth: 5035 ft

Grain size: 10 to 50 μm (quartz)

Grain shape: Subrounded to subangled (quartz)

Sorting: Moderate sorted (quartz)

Matrix: Dolomite

Diagenesis: The sample is mainly composed by sandy-dolomite (dolostone). The presence of quartz and felspar minerals and locally anhydrite as mineral and cement is highlighted. The porosity is due to the dissolution of quartz and felspar and averages 2%.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130673

Well: J-ROC 1

Depth: 5037 ft

Grain size: 10 to 50 μm (quartz)

Grain shape: Subrounded to subangled (quartz)

Sorting: Moderate sorted (quartz)

Matrix: Dolomite

Diagenesis: The sample is composed by interbeds of sandy-dolomite and sandstone. The presence of quartz and felspar as mineral and anhydrite as mineral and cement in sandstone part is highlighted. The iron oxides such as pyrite are as isolated mineral and thin layers. The contact between quartz grains is long, tangential, and separated by anhydrite. The porosity is null.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130674

Well: J-ROC 1

Depth: 5041 ft

Grain shape: Subrounded to subangled

Grain size: 10 to 50 μm (quartz)

Sorting: Moderately sorted

Cement: Anhydrite

Diagenesis: This sample is a sandstone. It is composed by quartz and feldspar with the presence of anhydrite as cement. The presence of iron oxides is observable. Dolomite is present as thin layers in this sample including some quartz minerals. The contact between grains is tangential and mainly separated by the anhydritic cement. The observed porosity is almost null.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130675

Well: J-ROC 1

Depth: 5046 ft

Grain size: 10 to 30 μm (quartz)

Grain shape: Subrounded to subangled (quartz)

Sorting: NA

Matrix: Dolomite

Diagenesis: The sample is sandy-dolomite. This sample is mainly composed by dolomite, quartz, and feldspar. The presence of iron oxides. The porosity is due to the dissolution of quartz and felspar and averages 8%.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130676

Well: J-ROC 1

Depth: 5048 ft

Grain size: 10 to 30 μm (quartz)

Grain shape: Subrounded to subangled (quartz)

Sorting: NA

Matrix: Dolomite

Diagenesis: The sample is sandy-dolomite. This sample is mainly composed by dolomite, quartz, and feldspar. The presence of iron oxides. The porosity is locally due to the dissolution of quartz and felspar and averages 3%.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130677

Well: J-ROC 1

Depth: 5052 ft

Grain size: 10 to 300 μm

Grain shape: Subrounded to subangled

Sorting: Moderately sorted

Cement: Dolomite

Diagenesis: This sample is a coarse sandstone. It is composed by quartz and feldspar. The presence of dolomite and locally anhydrite is noticeable as cements. The quartz minerals show overgrowth and sometimes corrosion. The presence of iron oxides is seen. The contact between grains is tangential and separated also by the dolomitic and anhydritic cements. The observed porosity is intercrystalline and due also to the dissolution of feldspar and quartz and averages 10%.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130678

Well: J-ROC 1

Depth: 5056.5 ft

Grain size: NA

Grain shape: Needles

Sorting: NA

Matrix: Anhydrite

Diagenesis: This sample is an anhydrite. It is composed mainly by anhydrite minerals. The presence of quartz and iron oxides is observable and highlight the filled fractures. The observed porosity is almost null.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130679

Well: J-ROC 1

Depth: 5071 ft

Grain size: 10 to 50 μm (quartz)

Grain shape: Subrounded to subangled (quartz)

Sorting: Moderate sorted (quartz)

Matrix: Dolomite

Diagenesis: The sample is mainly composed by an upper part sandy-dolomite and lower par pure anhydrite. The sandy-dolomite is composed by interbeds of sand, dolomite, and anhydrite and clay as cements. The anhydrite part is composed exclusively by anhydrite mineral. The presence of iron oxides is highlighted. The porosity is due to the dissolution of quartz and felspar and averages 5% in the upper part.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130680

Well: J-ROC 1

Depth: 5075 ft

Grain size: 10 to 50 μm (quartz)

Grain shape: Subrounded to subangled (quartz)

Sorting: Moderate sorted (quartz)

Matrix: Dolomite

Diagenesis: The sample is mainly composed by sandy-dolomite. The sandy-dolomite is composed by quartz, dolomite, and anhydrite and clay as cements. The presence of iron oxides is highlighted. The porosity is due to the dissolution of quartz and felspar and averages 5%.