THIN SECTIONS DESCRIPTION. WELL J-ROC 1

Opeche Formation (Core Plugs)

Energy & Environmental Research Center | 15 North 23rd Street, Stop 9018 | Grand Forks, ND 58202-9018

## PHOTOMICROGRAPHS

6 thin sections were described in Opeche Formation in the well J-ROC 1. The description was carried out based on the grain size, shape, cement/matrix type, mineral components, diagenesis, and depositional environment. An average of 30 photos were taken for each thin section where some of them were selected to illustrate the main mineral components, fractures, porosity, and type of contacts between grains in each sample.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130614

Well: J-ROC 1

Depth: 4692 ft

Grain size: 20 to 200 μm

Grain shape: Subangled to subrounded (quartz)

Sorting: Poorly sorted

Matrix: Clay

Diagenesis: The sample is mainly composed by clay. The presence of quartz, feldspar, and iron oxides is highlighted.  in addition to the presence of anhydrite clasts. The porosity is very low and averages 2 %.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130615

Well: J-ROC 1

Depth: 4705 ft

Grain size: 20 to 100 μm (quartz)

Grain shape: Subangled (quartz)

Sorting: Poorly sorted

Cement: Clay

Diagenesis: The sample is shaly-sandstone. It is mainly composed by quartz, feldspar, and clay. The presence of anhydrite as mineral and local cement and iron oxides is highlighted. The observed porosity is almost null.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130616

Well: J-ROC 1

Depth: 4714 ft

Grain size: 20 to 350 μm

Grain shape: Subangled (rare)

Sorting: Poorly sorted

Cement: Clay

Diagenesis: The sample is composed by alternance of sandy-shale and sandstone. It is mainly composed by quartz, dolomite, clay, feldspar. The porosity is present in the sandstone layers and due to the dissolution of quartz and feldspar and averages 8 %.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130617

Well: J-ROC 1

Depth: 4728 ft

Grain size: 20 to 350 μm (quartz)

Grain shape: Subangled to subrounded (quartz)

Sorting: Moderately sorted

Cementation: Anhydrite and clay

Diagenesis: The sample is mainly composed by quartz and feldspar (sandstone). The presence of clay and anhydrite as cements is highlighted.  In addition to the presence of rock fragments, anhydrite, feldspar, and iron oxides. The contact between grains is tangential to long and sometimes the grains are separated by the anhydritic and clay cements. The porosity is due to the dissolution of quartz and feldspar and averages 8 %.

FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130618

Well: J-ROC 1

Depth: 4731 ft

Grain size: 20 to 350 μm (quartz)

Grain shape: Subangled to subrounded (quartz)

Sorting: Moderately sorted

Cementation: Anhydrite and clay

Diagenesis: The sample is mainly composed by quartz and feldspar (sandstone). The presence of clay and anhydrite as cements is highlighted.  In addition to the presence of rock fragments, anhydrite, feldspar, and iron oxides. The contact between grains is tangential to long and the grains are separated by the anhydritic and clay cements. The porosity is due to the dissolution of quartz and feldspar and averages 10 %.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130619

Well: J-ROC 1

Depth: 4735 ft

Grain size: 20 to 350 μm (quartz)

Grain shape: Subangled to subrounded (quartz)

Sorting: Moderately sorted

Cementation: Anhydrite and clay

Diagenesis: The sample is mainly composed by quartz and feldspar (sandstone). The presence of clay and anhydrite as cements is highlighted.  In addition to the presence of rock fragments, anhydrite, feldspar, and iron oxides. The contact between grains is tangential to long and the grains are separated by the anhydritic and clay cements. The porosity is due to the dissolution of quartz and feldspar and averages 8 %.