THIN SECTIONS DESCRIPTION. WELL J-ROC 1

Piper Picard Formation (Core Plugs)

Energy & Environmental Research Center | 15 North 23rd Street, Stop 9018 | Grand Forks, ND 58202-9018

## PHOTOMICROGRAPHS

6 thin sections were described in Piper Picard Formation in the well J-ROC 1. The description was carried out based on the grain size, shape, cement/matrix type, mineral components, diagenesis, and depositional environment. An average of 30 photos were taken for each thin section where some of them were selected to illustrate the main mineral components, fractures, porosity, and type of contacts between grains in each sample.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130608

Well: J-ROC 1

Depth: 4622 ft

Grain size: 5 to 50 μm (quartz)

Grain shape: Subangled (quartz)

Sorting: Poorly sorted

Matrix: Clay

Diagenesis: The sample is a shaly-sandstone. It is mainly composed by quartz, feldspar, and clay. The presence of anhydrite, dolomite, and iron oxides is highlighted. The presence of few opened fractures is underlined. The observable porosity is very low and averages 2%.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130609

Well: J-ROC 1

Depth: 4636.5 ft

Grain size: NA

Grain shape: NA

Sorting: NA

Matrix: Anhydrite

Diagenesis: This sample is an anhydrite. It is composed by anhydrite. The presence of dolomite, clay and iron oxides is observable and highlight the filled cracks and fractures. The observed porosity is null.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130610

Well: J-ROC 1

Depth: 4647 ft

Grain size: NA

Grain shape: NA

Sorting: NA

Matrix: Anhydrite

Diagenesis: This sample is an anhydrite. It is composed by anhydrite minerals. The presence of dolomite and iron oxides is observable and highlight the filled cracks and fractures. The observed porosity is null.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130611

Well: J-ROC 1

Depth: 4662 ft

Grain size: NA

Grain shape: NA

Sorting: NA

Matrix: Anhydrite

Diagenesis: This sample is an anhydrite. It is composed by anhydrite minerals. The observed porosity is null.

.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130612

Well: J-ROC 1

Depth: 4676 ft

Grain size: NA

Grain shape: NA

Sorting: NA

Matrix: Clay

Diagenesis: The sample is mainly composed by mixture of anhydrite, quartz, dolomite, feldspar, and clay minerals and clasts. The presence of iron oxides is highlighted. The presence of few opened fractures is underlined. The porosity is almost null.

## FOCUSED PHOTOMICROGRAPH OF INTEREST

## Star #: 130613

Well: J-ROC 1

Depth: 4681 ft

Grain size: NA

Grain shape: NA

Sorting: NA

Matrix: Anhydrite

Diagenesis: The sample is mainly composed by anhydrite. The presence of iron oxides is highlighted. The presence of closed fractures filled by quartz, feldspar, dolomite, and clay and iron oxides is underlined. The porosity is almost null.