CFU 31F-2,3

Cranfield Unit

Franklin County, Mississippi

Prior to rig move on, install ±60’ of 20” conductor pipe.

Move in rig up. .

Drill 17½” hole to ±2,200’MD/TVD with spud mud.  Limit penetration rates that will not overload solids control equipment. Circulate & condition mud. Short trip. Circulate & condition mud. POOH measuring drillpipe. Prepare to run 13-3/8” surface casing.

Run/cement 13-3/8” surface casing to 2,200’. Install two joints between float shoe & float collar.  All couplings should be thread locked.  Utilize bow-type centralizers: three on bottom joint, two on second joint from shoe and one ±20’ below ground. Circulate & condition well prior to cementing. Cement casing to surface using 100% excess in the open hole section.  Displace cement with the rig pump.  Bump plug and ensure that the floats are holding.  Perform top job.

Install 13-3/8” SOW x 13-5/8” 3M “A” section. Note: Sandia will purchase wellhead. Test void to 50% of Collapse for 30 minutes.

N/U Rig BOP stack, testing same to 250/3,000 psi (Annular: 250/1,000 psi).

Note: 13-5/8” BOP’s may be rental item.  Test casing to 1500 psi for 30 minutes. Record on chart. If pressure declines by more than 10%, consult office.  (Test kelly and safety valves simultaneously.)

PU 12-1/4" PDC bit, bit sub, 7-1/2” DC’s, & HWDP on 4½" drill pipe.  Clean out to within 10' of float collar and pressure test casing to 1000 psig surface pressure for 30 minutes.  Drill out float equipment and 10' of formation.  Pressure test casing seat to 10.5 lb/gal equivalent.

Drill ±120' of new hole at reduced RPM and weight on bit until the stabilizers are below the casing shoe. Continue to drill hole to ±10,732’MD/TVD, taking inclination surveys every 500’ and drilling cores as per attached Coring Program.

Make wiper trips as necessary. Make short trip. Circulate and condition mud for logging. POOH to log.

Note: Run solids control equipment to keep low gravity solids to ±5%. Maintain weight with barite additions. Below 7,000’ maintain fluid loss 6 - 8. Below chalk monitor HPHT fluid loss.

Run open hole logs per geologist's recommendation from TD to surface casing.

Note: Verify injection interval top perf and monitor sand depths for casing BHA spaceout

Make wiper trip and Circulate and Condition mud.  POOH.

Run 7-5/8” fiberglass and 7” steel casing with external lines as per Sandia Casing Running Procedure.

Circulate open hole and cement well per program. Displace with water or completion fluid,  Bump plug and ensure that floats are holding.  WOC as required.

ND BOP’s Install 11” 3M x 7-1/16” 5M csg head, testing void to 50% of collapse for thirty minutes. Note: Sandia will purchase wellhead.  N/U Dry hole tree. Clean tanks , RDMO Drilling rig.